Assignment Cover Sheet

| Student Name: | Oluwakayode Seun Abiodun |
| --- | --- |
| Student Number: | 1256 |
| Course: | Diploma in Construction Management |
| Assignment NO: | 5 |

Answer for Question 1 - Q: What do you understand by the term ‘Site Organization? A: Site organization is the production or orderly, logical and practical site layout plan that will show proposed locations of all facilities, accommodation and plant to secure optimum economy, efficiency and safety during construction. The major items to be considered in the site organization are access roads, site accommodation for office, personnel and labour, site electricity, storage of materials, plants and movement of plants and so on - Q: Highlight the importance of preparing the site road suitable for the movement of heavy construction equipment? A: Preparing suitable road for movement of heavy construction equipment while facilitate efficient and economic operation of the machines on site - Q: List the precautions to be taken for the storage of cement at a construction site? A: 1. Storage of cement is to be provided in a shaded area or building which is dry, leak proof and as moist as possible. 2. Cement must be kept in a well-ventilated building to protect them from dampness. 3. Cement bags should be stacked off the floor on wooden planks in such a way as to keep them 150-200mm clear from the floor and minimum space of 450mm should be left around between exterior walls and the stacks. 4. To prevent the possibility of lumping up under pressure, the width of a stack should be restricted to 4 bags length or 3m and height of stack to 15 bags. 5. Cements should not be stored for more than two months before use in humid climate. Storage during monsoon or in humid area should be enclosed completely with water proofing membrane. - Q: What will you do to prevent scaling and rusting of structural and reinforcing steel? A: To prevent scaling and rusting of structural and reinforcing steel, the best practice is to coat the steel with cement wash before staking - Q: How does exit control help in site production? A: Exit control helps to necessitate maximum security on site. It controls and monitors the movement of vehicle and personnel in and out of the site which helps in protection of assets as well as reduces threat from thief and trespassers. Answer for Question 2 Q: Discuss the advantages of providing fences for construction sites. A: Fences provide a degree of protection to a construction sites. Fences fulfils two main functions which are; - It defines the limit of the site or compound, provide protection to assets and personnel - It also acts as d deterrent or physical barrier to any trespasser or thief that may want to invade the site. A fence can be constructed to provide physical barrier of solid construction or a visual barrier of open work construction. The type of fences to be use will depend upon the degree of security required, cost implication, type of neighborhood and duration of contract. Answer for Question 3 Q: How is the provision of site electricity different from building electricity? A: Construction site electricity differs from building electricity because they are mainly outdoor electrical installations which are open to sky or partially covered. The power that is provided by the site electricity to drive small and large equipment and plants are generally unprotected from environmental hazards compared to installations in buildings. The is also special consideration required as most of the installations that use site electricity are characterized by frequent modification which are not most required in building.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | Oluwakayode Seun Abiodun | Date | 05-08-2014 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |