| Student Name: | Vismantas Mikstas |
| --- | --- |
| Student Number: | 1302 |
| Course: | Diploma in Construction Management |
| Assignment NO: | 3 |

- Give short answers for the following questions: - What is meant by estimating and why are estimates necessary? In construction estimate means working out approximate requirement of various resources required for constructing structure. Resources for construction constitute materials, labor (human efforts), equipment and time. All these resources are closely linked with money. It is therefore, customary to estimate the money required for constructing a structure. Estimating is therefore, very commonly understood to be working out the approximate cost they will have to be spent for creating a structure. This cost is worked out before the construction is started. The actual cost is known only-after the structure is completed. A good estimate should be as close to the actual cost of construction. The owner wants to know the cost of structure to provide necessary funds, and to compare the offers given by tenders, if the owner intends to entrust the work to a contractor. The owner intends to select the most competitive one as he is interested in constructing the structure at the lowest price. A contractor is interested in the project estimates to ensure that he can earn a reasonable profit, and he can organize various resources to complete the construction. The consultant or architect also have interest in the estimates of cost so he could charge appropriate consultation fee to the client. Or assist the client in selecting the contractors. - What is bill of quantities? A bill of quantities is a document used in tendering in the construction industry in which materials, parts, and labor (and their costs) are itemized. It also details the terms and conditions of the construction or repair contract and itemizes all work to enable a contractor to price the work for which he or she is bidding. Bill of quantities is a statement showing the item (or activities) with their quantities and unit of measurement. This statement supported by the specification assists planning, scheduling and field execution of the project. If the construction is to be done departmentally the approved project estimates serve this purpose. These estimates will have to be supplemented detailed working drawings and estimated as and when required during the course of construction. These estimates will be, normally within the provisions of the sanctioned estimates and have to be approved by the competent authority. Bills of quantities are prepared by quantity surveyors and building estimators. - Explain the term approximate estimates and how they are prepared? Quick method of estimating approximate cost of a new structure consists of two steps: 1) To work out the total units involved in the structure based either on physical parameter e.g. square meters of plinth area, cubic meters of volume of building, length of road or service/utility rendered by the project e.g. area irrigated in hectares, number of students in a school or college; number of beds in a hospital etc. 2) Multiply the above by the known cost of the structure per unit. To enable the estimator to arrive at the reliable unit rates to be used in approximate estimating it is necessary for the site supervisors to keep cost data sheets on the work site on construction operation (during execution of a project) detailing total cost of materials, labour, equipment etc. The resultant unit cost per structure is then worked out. In addition to being useful in checking the actual cost estimated at the starting of the project, this unit rate also serves as the basis of preparing an approximate estimation of a similar project in future. It is however, essential for an estimator to take into account: - variations in the prices of materials, labour and equipment that might have taken place since the completion of the project forming the basis of unit rate, - difference in the site conditions, design methods, materials, method of construction etc. for the new structure, - Possibility of variations in the labour and equipment outputs likely to occur due to change in conditions and place when new structure will be constructed. Reliability of the approximate estimate depends on accuracy of cost data maintained, and price adjustments done to account for the change in the conditions mentioned above. - What is meant by project budget? How it is prepared? Project budget can be defined as a plan for future against which the progress actually achieved is measured. Planning for a construction project is done in the form of construction programme. It shows the sequence in which the activities will be taken up. Before starting an activity it is necessary to initiate action to organize the resources required by the activity. Thus if the date on which the concrete is to be poured is decided and if takes up a week to place orders for resources like cement, aggregates and get them on site in time to start pouring concrete on the scheduled date the procurement of the resources has to be initiated one week before the date of pouring. Expenditure on concreting activity will have to be incurred ahead of the actual-concrete. It will be obvious from this the construction programme prepared in terms of start and completion of activities will have to be translated into work related to activities in progress and the cost thereof, and work related to activities to be taken up in future 'and costs thereof. This can be illustrated with a case: - Construction supervisor has to continue masonry work next day by employing labour. He will have to pay labour wages. - He has planned to start concreting work for which he has to convey material to the site work. He has to spend on conveyance chargeable to the item of concrete. - He has to place order for certain materials for the work to be started next week. He has to pay advance with the orders. This expenditure is chargeable materials for the concerned item. It is thus possible to prepare detailed programme of expenditure to he incurred on any day together with the items (and sub-items) to which these items are chargeable. This constitutes the project budget for the field staff. - Explain the term administrative approval and technical sanction Project report together with the estimate of cost must receive the approval of the competent authority. For the construction projects two types of approvals are sought for: 1) Administrative Approvals. This is the first stage approval. It is obtained from the competent authority of the user department. Thus when Health Department wants to construct a hospital it approaches the Public Works Department to take up construction of the hospital building. PWD carries out feasibility studies. Prepares the layout and rough drawings and specifications on the basis of which approximate cost estimates are prepared as explained in the earlier paragraph. These approximate and drawings are forwarded to the user department for approval. This is known as administrative approval and is an indication of acceptance of the proposals by the user department and its commitment to make the funds for the project available by PWD. 2. Technical Sanction On receipt of the administrative approval the PW Dept. takes up the work of detailed design, drawings and estimates as a first step to take up construction. Detailed report comprising designs, drawings, specification and estimates of cost have to be approved by the competent authority of PW Dept. - How are the estimates made for the following? - A material rate Estimator should know the quantity of material (of specified requirement) for each unit item. This is worked out on the basis of specifications of the item. Price of the material is obtained from the quotations from the suppliers. Over and above the price of procuring the material it is necessary to add expenditure to account for: - transport of the material, - loading and unloading, - handling on site, - storing, watch and ward, - Wastage to cover breakage, losses, inevitable pilferages etc. which may vary from 2% to about 10%. Information on the above points enables estimator to build up the cost of materials required for unit item. - Labour rate Labour rate for a unit item depends on the category and number of labour employed. Category depends on the trade (carpenter, mason, bar bender etc.). Number of labour depends on the quantity of item and output of labour per unit time - a working day. Labour output is the most uncertain plan. It depends on the factors like skill of the worker, site organization, weather conditions and many other factors which are often beyond the control of supervisor. Records of labour output for all the trades have to he kept on site for many years noting varying conditions under which they are observed. A comprehensive list of unskilled, semi-skilled and skilled time is compiled. These lists are helpful in working out the labour components (in terms of labour days) for each unit Item. The estimator has then to calculate cost of the labour per head per day i.e. the labour rate. This depends on the following factors: - Basic wage primarily governed by the prescribed minimum wages or fair wages. - Payment for inclement weather when the labour reports to work but cannot be assigned any work or can do work only for a part of the day due to unfavorable weather or heavy rains. - Paid holidays. - Expenditure incurred on travel, labour camps. - Overtime payment. - Sickness benefits. - Insurance. - Any other payment as per statutory requirement. - Conveyance of labour. - Equipment rate On the lines similar to the labour rate, equipment rates are also calculated. Equipment hours required for an item depend on the quantity of the item and the output of the equipment under the conditions of use. The equipment rate can be worked out either on the basis of equipment hour or the unit output of the item. Equipment rate is based on - Cost of Capital Investment. - Depreciation. - Cost of Fuel & Lubricants. - Repairs & Maintenance. - Salaries of Operators & Helpers. - Allowance for possible idle time. - Insurance. - Any other related expenses. On the basis of cost of materials, labour rate and equipment rate the unit rate for each item is worked out, which is then used for working out the cost of the item. For items which are not amenable to accurate measurement lump sum provision is made on the basis of past experience or the judgment of the estimator about the work content. - Discuss estimating procedure in details. Accurately forecasting the cost of future projects is vital to the survival of any business or organization contemplating future construction. Cost estimators develop the cost information that business owners or managers, professional design team members, and construction contractors need to make budgetary and feasibility determinations. From an Owner's perspective the cost estimate may be used to determine the project scope or whether the project should proceed. The construction contractor's cost estimate will determine the construction bid or whether the company will bid on the construction contract. From the drawings estimator prepares list of the activities to be performed. This constitutes the list of items which further forms the basis for bill of quantities. For each item a suitable unit is chosen which will be the basis of measurement for estimating and later on for the basis for tendering and making payment to the contractor. In addition to these units provisions are made on lump sum/job basis for those items which are not on measurement wise. For all the items listed as above the appropriate dimensions are taken off the drawing and the quantities are worked out in the respective units with due allowances for deductions for the portions where the work is not done e.g. openings for doors and windows. The result of the two above steps is a list of items together with their quantities and the units. The items are further priced at the rates per unit of item which are built up on the consideration of resources required for unit of items. Each item of the construction requires: materials, labour and equipment. It is necessary to build up material rate, labour rate and equipment rate for each item. Estimator should know the quantity of material for each unit item. This is worked out on the basis of specifications of the item. Price of the material is obtained from the quotations from the suppliers. It is necessary to add expenditure to account for: transport of the material, loading and unloading, handling on site, storing, watch and ward, and wastage to cover breakage, losses, inevitable pilferages etc. Information on the above points enables estimator to build up the cost of materials required for unit item. Labour rate for a unit item depends on the category and number of labour employed. Number of labour depends on the quantity of item and output of labour per unit time-a working day. Labour output is the most uncertain plan. It depends on the factors like skill of the worker, site organization, weather conditions and many other factors which are often beyond the control of supervisor. The estimator has then to calculate cost of the labour per head per day. On the lines similar to the labour rate, equipment rates are also calculated. Equipment hours required for an item depend on the quantity of the item and the output of the equipment under the conditions of use. The equipment rate can be worked out either on the basis of equipment hour or the unit output of the item. Equipment rate depend on the project conditions to determine the correct size or capacity of equipment required to perform the work. When interfacing with other equipment, cycle times and equipment capacity control the costs on the project. Costs will also differ if the equipment is owned by the contractor as opposed to rented ones. Equipment rate also depends on cost of capital investment, depreciation, cost of fuel & lubricants, repairs & maintenance, salaries of operators, allowance for possible idle time, insurance and any other related expenses. On the basis of cost of materials, labour rate and equipment rate the unit rate for each item is worked out, which is then used for working out the cost of the item. For items which are not amenable to accurate measurement lump sum provision is made on the basis of past experience or the judgment of the estimator about the work content. The cost worked constitutes the cost of the work portion. To this are added the following costs: - Expenditure on surveys, data collection, design, drawings and estimating; - Land acquisition; - Establishment required during the execution of work; - Allowance for unforeseen works that may be found necessary in the course of execution; - Overhead expenses to be incurred on the site as well as in head office. All the costs mentioned above add up to the total cost of the project. Project report together with the estimate of cost must receive the approval of the competent authority.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | V.Mikstas | Date | 22/11/14 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |