Assignment Cover Sheet

| Student Name: | Suckson Makwasha |
| --- | --- |
| Student Number: | 1249 |
| Course: | CONSTRUCTION MANAGEMENT |
| Assignment NO: | 4 |

1. I). Site organization means is layout that is logical, orderly and practical and is it shows proposed positions of facilities such as accommodation, ablutions, site offices and site location. ii). The importance of preparing the site road suitable for the movement of heavy construction equipment, this will help for easy access for delivery vehicles, for efficient movement of haulage trucks to spoil sites, suitable site roads also helps to protect the environment because vehicles will use designated routes whereas if there are no roads drivers tend to use any available short routes without considering environment in an area. The use of un designated routes may lead to damage or destruction of some protected species. III). Precautions to be taken for the storage of cement at a construction site are: Must be stored in a shade, dry and area that does not allow moisture penetration In a ventilated area and must not be stored direct on the ground as it would become Damp. Cement should not be stored for more than two months before use. IV). Structural and reinforcing steel must be coated with cement wash before staking to prevent scaling and rusting. V). Exit control help in site control, it limits the movement of vehicles on site as they are controlled when they exit. Exit control on site also help to manage risks of accidents and damages to the environment and surrounding structures. 2).The advantages of providing fencing for construction sites are: a). It protects vandalism of plant and theft of materials within the site. b). It restricts random access into site c). Fencing protects animals and people from falling into deep excavation during works. d). Fencing demarcates the site and will help to see the extend of the site e). Acts as a deterrent to the would be trespasser or thief. f). The other advantage is that fencing helps to control in and out vehicles since there will all use one entrance and exit. 3). Electricity supply for site is always an outdoor electrical installations open to sky or Partially covered where as for buildings electricity is an indoor and always covered. Site electricity is generally unprotected from environment hazards where as for For buildings is protected fully from environment. For buildings there are special considerations during installation whereas site electricity Its done to suit requirements for that time only without due considerations of future.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | S. Makwasha | Date | 17/06/2014 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |

r