Assignment Cover Sheet

| Student Name: | Mr Tamas Galler |
| --- | --- |
| Student Number: | 1323 |
| Course: | Diploma in Construction Management |
| Assignment NO: | 11 |

Student name: Tamas Galler Student no: 1323 Assignment no: 11 Page no: 1 - Give short answers for the following questions: - What are the economic objectives of quality control how are they achieved? The objective of any manufacturing process or construction is to obtain the item or structure at minimum cost while maintaining the quality of the product so that the life of the product is extended as much as possible. The economic objectives of quality control by the contractor are as follows: (a) saving through the reduction of rejects, (b) savings through closer approach to design standards rather than greatly surpassing them, (c) savings to the owner and user through the creation of a better product at no added cost. A very important feature of a good quality control programme is that better quality and lower costs go hand in hand. All this is possible if the following categories of quality control are considered. These categories are as follows: (a) Failure costs, which are caused by defective materials, and products that do not meet the owner's quality specifications. They include such loss elements as re-work scrap, field complaints due to non-performance as per requirement, and spoilage. (b) Appraisal costs, which include the expenses for maintaining quality levels by means of formal evaluation of product quality. This involve such cost elements as inspection, testing, outside certification with quality audits. (c) Prevention costs, which are for the purpose of keeping defects from occurring in the first place. Included here are such elements as quality control engineering, employee quality training, and the quality maintenance of products and plants. - What are the features of field quality control? The basic instruments, equipment and facilities required for a field quality control programme will depend on the mission and services expected from the quality control team or laboratory. The problems facing most laboratory managers are those of justifying the necessity for equipment to satisfy the occasional need, evaluation of its projected utilisation, and its availability in other sections of the project. In addition, they must consider effects of non-performing a specific test and die floor space and other services required for operating the equipment or tool. Student name: Tamas Galler Student no: 1323 Assignment no: 11 Page no: 2 The normal support efforts required of a field quality laboratory for product control can usually be well planned and managed with a minimum of effort after the initial planning stage when occasional peaks occur in the total work load, management by priority is a necessity. Each job must be evaluated and assigned a priority rating to ensure that the most important jobs are given precedence. Field quality control involves: (a) the desire to do well all of those things which need to be done, (b) seeking out new and better ways to accomplish a given task, (c) the intelligent application of experience, (d) pride in one's effort, (e) favourable consideration for future business to the contractor, and (f) happiness and satisfaction to the owner in knowing that he is getting what he is paying for. - How can contractor quality control be implemented effectively? (a) The most important ingredient is that of attitude. If a contractor is conscientious, it is more likely that quality control will work. (b) The calibre of quality control personnel. Where quality control personnel are experienced and capable, the quality of work is good. Men who have previously worked for public agencies are found to be most effective. College graduates are also found to be effective in supervising materials testing. (c) The owner must be clear what he expects of the contractor's quality control. The owner must spell out the quality control requirements. He must also specify the qualifications and number of quality control personnel the contractor must hire. These measures help assure an effective program and also put all bidders on equal footing. (d) The need to separate quality control and production. In the contractor's Organizational setup, the quality control function must report directly to the project manager. If quality control reports to the construction supervisor instead of the project manager, production may prevail but at the expense of quality. - Why are specifications important? Specifications are necessary to explain to the contractor the quality of workmanship which will be required of the contractor who is to build a construction project. Workmanship meant to denote the contractor's operations in the shop or field rather than the materials used by him in the performance of the contract. The conventional specification generally controls quality by setting a series of absolute max-min limits on the physical properties for the materials, processes and performance of the item. Such limits many times do not adequately recognize the variability of materials and processes, the acceptance of materials under this method is based upon the assumption that representative samples provide a satisfactory picture of the characteristics of the material provided. If any test indicates the material or component is outside the specified limits, the material represented by the sample should be rejected. Another important factor, in evaluating the quality of construction components, is the number of samples used for the evaluation. The application of statistics will permit determination of the probability of accepting a lot or component which does not in fact meet the specification requirements. Conversely, the probability of rejecting a lot or component which actually does meet the requirement can also be determined. Thus realistic specifications can be developed; however, it is necessary that the designer decide exactly what physical properties and characteristics are required and what cannot be tolerated. Student name: Tamas Galler Student no: 1323 Assignment no: 11 Page no: 3 - What are incentives and penalties in specifications? Incentives and penalties do not serve the same purpose in specifications. From a psychological point of view, they have definitely different effects on people. If we desire to bring about equitable specifications and eliminate the adversary owner-contractor relationship, incentives should be used and penalties avoided. Incentives are used for motivating the work force to bring about a desired action. In incentive specifications, there is no thought of accepting below the required level of quality (with its tolerance), and the incentive is to motivate the contractor to control his process in such a manner so as to reduce his variability in attaining the desired quality. Incentives work in motivating 100 percent of the time. Incentives usually bring about positive and desirable reactions in a person. Penalties are for punishment having failed to achieve the desired end or for doing something undesirable. In penalty based specifications, the contractor knows that he can have below quality levels accepted by paying a penalty, and he can add into his price this anticipated penalty, thus costing the owner more than the incentive bid price, and the owner may land up with a lower quality than specified. Penalties are paid for by the owner 100 percent of the time. Penalties have always resulted in negative and undesirable reactions. - How can you achieve quality control in the field? Quality control in the field can be achieved by (a) developing within the owner's organisation the technical and practical expertise necessary to define the quality requirement for each facility, in terms that can be understood by the design organisation and field management, (b) producing clear and accurate models, drawings, details, and specifications in the design office, with proper considerations for construction in the field, (c) involving contractor's field management, personnel in construction reviews, as the design progresses, (d) insisting on the procurement of quality materials of construction, (e) emphasising the need for careful protection of materials and equipment in the field, in storage and handling and after installation; and by providing the contractor with the information necessary to accomplish this, (f) encouraging studies to determine the best methods of construction and the right tools and equipment for the job, (g) keeping abreast of the latest developments in equipment and techniques for determining quality in the field and seeing that the contractor is aware of these where appropriate, (h) insuring that he contractors recognize responsibility for quality control function and provide proper staff to make it happen, (i) assisting construction establishing effective, on-site quality control programmes, (j) frequent visits to the field by technically competent experienced personnel, to review the contractor's quality control records and procedures; and to willingly assist in a team effort to solve problems when unusual conditions are encountered, (k) providing competent resident engineers who are prepared and authorised to exercise sound judgement when quality problems arise, Student name: Tamas Galler Student no: 1323 Assignment no: 11 Page no: 4 (l) requiring contractors to insure that every potential subcontractor thoroughly understands all of the quality requirements and objectives on his part of the work, before an award is made, (m) establishing a feedback to the design organisation that will make it possible to benefit, on future projects, from difficulties encountered in the field, due to particular design details, and (n) involving the appropriate design representative in the solution of problems encountered by the field personnel. - What are the advantages and disadvantages of contractor quality control? Advantages of contractor quality control are follows: (a) Testing and inspection is a necessary cost, and the money can be more efficiently managed by the contractor who directly control production. (b) The contractor is the only one who is in a position to anticipate and order the necessary adjustment in production to assure proper quality. (c) Quality is built into a product and not inspected into it. The best that an owner can do is to conduct an effective quality assurance program. Testing, inspection, evaluation, and appropriate action by the contractor is a means of building quality into the product. (d) If the contractor does his own testing, he is forced to know his product better, which in itself could result in an improved product, perhaps at a reduced cost. (e) Statistical specifications can be used more efficiently if integrated into the contractor's effort. By maintaining a quality control chart mounted or displayed so that everyone can see and evaluate the degree of control the contractor will encourage and simplify quality control as well as establish an easy means of communication concerning his quality control. (f) The department contract directly with a general contractor. The contractor may purchase from a supplier and do his own hauling, spreading, and compaction, or some other combinations of work items. Controversies may arise as to responsibility when material handled by more than one organisation does not meet the specifications. With the contractor responsible for quality control, the department no longer would need to arbitrate such controversies. (g) The owner's inspecting and testing forces would have more time to devote to broader construction problems, which should insure a better overall project. Disadvantages of contractor quality control are follows: (a) Small contractors may not have the necessary facilities to do their own quality control, this will necessitate the hiring of an outside agency, in remote areas, and this may be a problem and add to the cost of the project. (b) From the user's point of view, quality control by the contractor would seem more effective when an endpoint specification is available. Not all products have adequate endpoint specifications. (c) Satisfactory quality control by the contractor will only be achieved if it is handled by personnel trained in quality assurance procedures with sufficient authority to actually control the quality of the contractor's product. Such experienced persons are not many.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | Mr Tamas Galler | Date | 11.02.2015 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |