Assignment Cover Sheet

| Student Name: | Matthew Nott |
| --- | --- |
| Student Number: | 2905 |
| Course: | Diploma in construction management |
| Assignment NO: | 11 |

Marking Criteria:

We expect the learners to write minimum one well expressed point in three lines against each allocated mark. This means one needs to write 15 lines with 5 well expressed points to get high grades for a 5 marks question.

For high grades use examples and illustrations where appropriate.

i. What are the economic objectives of quality control and how are they achieved? The overall goal of a construction project is to build a structure at the lowest possible price, at the highest quality. This can be achieved if the following costs are minimized - Failure cost due to quality issues with materials or products which may result in rework, scrapping, and low productivity on the field or even spoilage of items. - Appraisal cost inherent in conducting quality control assurance using testing, inspection and other means of product evaluation. - Prevention costs associated with avoiding mistakes during construction. These costs are linked to personnel training, quality control engineering and maintenance of equipment. Some economic reasons for undertaking quality control include: - Cost savings by minimizing rejects or rework - Cost savings by keeping within boundary of design specifications and not extending beyond - Savings obtained by making quality products at no extra cost to the parties involved in the project When the contractor engages the services of well-trained supervisors, maintains a high standard of specifications and understands that good quality control is imperative to increasing profitability, savings will follow. ii. What are the features of field quality control? Field quality control requires the use of equipment, employee expertise and any other relevant facilities required to meet the contract quality measures. Lab managers evaluate equipment requirements and deployments during quality control checks. It is usually best to perform the quality control checks after peak periods of activity, when workers become more available. Field quality control usually involves: - The desire to provide an excellent service - Discovering the most effective way of performing tasks - Making use of experience to get things done - Being proud of effort one makes to accomplish tasks - Providing business opportunity for the contractor - The owner getting joy and satisfaction from having value for money iii. How can contractor quality control be implemented effectively? The contractor’s quality control plan is vital for assuring the project client that everyone involved in the project is aware of the steps required to produce a high quality product. Prior to the commencement of work it is not unusual for all involved parties to discuss the quality control plan together. Contractors are expected to keep accurate records of all quality checks and inspections as well as details of problems that have risen and their solutions. Daily progress reports are also presented to the owner on all aspects of the project. A key part of ensuring this paperwork is carried out as per the quality control plan is regular checks by the owners’ quality control team. These checks will be randomly carried out on a variety of different project areas, leading the contractors ensure that the correct checks and inspections are being undertaken on all aspects of the project. iv. Why are specifications important? Specifications are the guidelines dealing with the quality of work being completed onsite, using the right quality of materials. Limits are set on the physical properties of materials for quality control. If a quality control inspector finds that a particular material being tested falls outside the specification limits, further testing will be carried out. If successive tests also fail it can lead to the material being rejected. It is therefore important that specifications are followed at all times as material failures can lead to: - Safety risks involved with materials failing, which can range from minor (potholes in a road) to major (section of a bridge span failing) - Financial risks due to rework costs and potential penalties from the client - Public relations and reputation damages due to cutting corners or producing subpar quality products leading to potential loss of work v. What are incentives and penalties in specifications? Penalties and incentives are often included in contracts as safeguards against contractors rushing to complete the project, or from delivering a product of inadequate quality. Government and private contracts tend to be slightly different in this regard. It is not uncommon for government contracts to include incentives for delivering a project early and or liquidated damages in the form of penalties in the event of a program overrun. However in private contracts it is advisable to avoid penalties due to the potential strain it can place on the relationship between the client and contractor. Incentives can often provide additional motivation for the workforce and help to create a positive atmosphere on the work site. 2) How can you achieve quality control in the field? In order to achieve quality control in the field it is paramount to begin with careful planning. Careful planning will go a long way to achieving the best quality product at minimal costs. Developing a good quality control plan requires asking the following questions and finding the right answers: - Which aspects of quality work are required to be executed - When is the ideal time to execute each activity during construction - What methodology or procedure should be used to accomplish these activities - Who should be responsible for the work in the field - Where specifically, should the work be executed - What tools, equipment or instruments are required - What inputs are required to get the structure completed - What is the expected end result of the quality control testing - Would the results of the tests warrant the identification and routing of materials - Should records of action be kept and in what format In order to achieve effective quality control, the following provides a guideline on what actions to take: - Developing the necessary technical and practical expertise that will be required to specify quality requirements for each activity and facility - Creating clear and accurate designs, drawings, specifications and models which will translate into quality products on the field - Having the contractors field management personnel take part in the design process and quality control specifications - Spearheading procurement of only quality materials - Ensuring measures are in place to protect equipment and materials during all phases of construction - Researching effective methods of construction using suitable equipment and tools - Developing and using the latest quality control technology and knowing how to function effectively - Making contractors understand the need for quality control and making sure his staff are trained to ensure the highest quality on the field - Providing assistance to establishing quality control on the job site - Frequent visits to the site to ensure quality control programmes are being executed and also collaborating with field staff to solve quality control programs - Making available an engineer with enough experience to deal with quality control issues - Ensuring that the contractor explains to subcontractors all of their responsibilities and objectives of the project - Keeping effective records of issues and providing feedback to design engineers to help future projects avoid such issues - Getting design personnel involved in providing solutions to problems arising in the field of construction 3) What are the advantages and disadvantages of contractor quality control? Advantages - When testing and inspection are integral parts of the project, then the cost can be well managed by the contractor - The contractor is in a stronger position to anticipate problems and take the necessary steps to adjust the work to solve the problem - The contractor is better placed to build quality into the product than the owners quality control supervisor - Testing by the contractor can help them improve on their performance or efficiency at a lower cost - Integrating statistical specification into the contractors daily activities and displaying this information for other workers to view on a daily basis will yield better overall quality - Disputes between the contractor, suppliers and quality control department may be avoided if all quality issues are made the sole responsibility of the main contractor - There will be more time available to the owners quality control team to focus on solving more general problems on the site Disadvantages - The contractor may not have enough capacity to perform satisfactory quality control programs - Contractor quality control is only suitable when the end point specifications are available - Well trained quality control experts will be required to assist the contractor to meet quality control targets. But unfortunately, there are very few personnel with these skills.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | Matthew Nott | Date | 30/01/2017 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |