Assignment Cover Sheet

| Student Name: | Maung Maung Min Thaik |
| --- | --- |
| Student Number: | 3040 |
| Course: | Diploma in Construction Management |
| Assignment NO: | #4 |

Marking Criteria:

We expect the learners to write minimum one well expressed point in three lines against each allocated mark. This means one needs to write 15 lines with 5 well expressed points to get high grades for a 5 marks question.

For high grades use examples and illustrations where appropriate.

1. Give short answer the following questions: (20) i. What is environmental impact study or EIS? Developmental activities are accompanied by some positive, as well as, some negative effects on the surrounding ecosystem. It is therefore imperative for developers to consider factors which affect the environment during the early phase of planning. Site developers must take note of the following factors: - Air quality - Water quality and quantity - Solid wastes - Noise - Radiation from nearby radio-active sources - Hazardous conditions - Energy supply and natural resources depletion - Protection of environmentally critical areas i.e. flood plans, wet-lands, beaches and dunes, unstable soil, steep slopes, aquifer recharge areas - Land use in coastal areas - Redevelopment in built-up areas - Density and congestion mitigation - Neighbourhood character and continuity. ii. What is the basic criterion for judging the suitability of the site for construction? It is important to select the very best site for building a structure for a specific purpose because the location tends to have effect on: - Cost of the project - Accessibility - Structural techniques required - Extent and nature of site preparation needed iii. What is topographic data about? Visual inspection is used to study the basic land form of the site. Sometimes there is the need to also prepare detailed maps. When gathering topographical data, the following features should be considered: - Elevation above sea level - Orientation of the site (e.g., east to southeast, etc.) - Slopes in percentage (0 to 3%, 3 to 8% etc.) - Surface waters (such as ponds, streams, rivers, and drainage patterns) - Hills and valleys (visibility, etc.) - Hood plains, swamps, and quick sand, etc. iv. Which type of buildings should be constructed on rocky sites? Dams, high rise buildings and tunnels should be constructed on rocky sites. 2. Give the classification under which the following buildings fall: (10) a) Museum : Class D1 covers many ‘public’ services (which do not fall under Class A) b) Hospital: Class C2 does cover such types of premises, providing they are residential c) Hotels: Class C1 deals with hotels, boarding houses, guest houses. This does not include premises which offer care as part of their services. That is to say, these premises are ‘regular hotels’ open to the general public, rather than those for guests or residents with special needs. d) Library (public): Class D1covers many ‘public’ services (which do not fall under Class A) e) Departmental stores: For those within Class A1, the customers, in every case, should be “visiting members of the general public”.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | Maung Maung Min Thaik | Date | 1/19/2017 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |