Assignment Cover Sheet

| Student Name: | Jodie Cowen |
| --- | --- |
| Student Number: | 27943 |
| Course: | Preliminary Investigations, Location and Site |
| Assignment NO: | 4 |

Marking Criteria:

We expect the learners to write minimum one well expressed point in three lines against each allocated mark. This means one needs to write 15 lines with 5 well expressed points to get high grades for a 5 marks question.

For high grades use examples and illustrations where appropriate.

Please insert your completed assignment ( in word format) here: - What is environmental impact study or EIS? Any construction/development project will likely impact, and be impacted by, surrounding ecosystems. An environmental impact study (EIS) is a report/study which considers and assesses these likely impacts. An EIS will consider factors such as: air quality, water quality and quantity, noise, radiation from nearby radio-active sources, hazardous conditions, energy supply and natural resources depletion, protection of environmentally critical areas (e.g. wet-lands, beaches and dunes) and density/congestion mitigation and should be commissioned during the earliest stages of development planning. - What is the basic criterion for judging the suitability of the site for construction? The suitability of a site for a specific structure/construction project is important as site suitability will have an impact on the cost of the project, accessibility, the structural techniques required and the site preparation works needed. Basic criterion are factors which affect all types of construction projects and these include the availability of physical resources (e.g. water, power, materials), climatic considerations and physical factors (e.g. the contour and composition of the ground). - What is topographic data about? Topography is the study of the shape and features of the surfaces of the land. Topographical data is often collected to enable the preparation of detailed maps of construction sites and the immediately surrounding areas. Topographical data includes: elevation above sea level, orientation of the site, slopes, surface waters, hills and valleys, hood plains, swamps and quick sand. Topographic data should be used during the earliest stages of development planning to facilitate the design and layout of the development, pre-development ground work requirements and the setting of ground floor levels. - Which type of buildings should be constructed on rocky sites? It is essential to investigate the geology of a site (type of surface deposits, bedrock formation and the way they interact) because this will impact on the stability of the ground and the susceptibility of the ground the erosion. These factors will impact the foundation design required for various structures and the ease and cost of excavating the ground for all below ground works (including but not limited to the construction of foundations). Rocks are classified based on their mode of formation (i.e. metamorphic, igneous, and sedimentary) and different rock classifications have different levels of permeability (metamorphic rocks such as sandstone are highly permeable when compared to igneous rocks such as granite). Site geology is a particularly relevant consideration when assessing the feasibility of large scale construction projects such as dams and tunnels. For example, a damn cannot be built within or upon rocks pervious to water (Preliminary Investigations, Location and Site, 2018). 2. Give the classification under which the following buildings fall: a) Museum – a museum has a Class D1 (non-residential institutions) classification. b) Hospital - a hospital has a Class D1 (non-residential institutions) classification unless it is a residential hospital in which case it would have a C2 classification. c) Hotels - a hotel has a C1 classification. d) Library (public) – a library has a D1 classification (non-residential institutions) classification. e) Departmental stores – Departmental stores have an A1 classification (Preliminary Investigations, Location and Site, 2018).

Student Statement: By submitting this assignment, I confirm that this is my own work.

| Student Signature | Jodie Cowen | Date | 27 May 2019 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |