Assignment Cover Sheet

| Student Name: | Amro M Fadly |
| --- | --- |
| Student Number: | 23258 |
| Course: | Construction Management |
| Assignment NO: | 4 |

1. Give short answer the following questions: (20) i. What is environmental impact study or EIS? EIS is the study of project factor effects (negative and positive) on the environment and echo system, prior to start, such as: - Air quality - Water quality and quantity - Solid wastes - Noise - Radiation from nearby radio-active sources - Hazardous conditions - Energy supply and natural resources depletion - Protection of environmentally critical areas i.e. flood plans, wet-lands, beaches and dunes, unstable soil, steep slopes, aquifer recharge areas - Land use in coastal areas - Redevelopment in built-up areas - Density and congestion mitigation - Neighborhood character and continuity The study is used to evaluate the consequences of the project and to proceed or not. ii. What is the basic criterion for judging the suitability of the site for construction? The suitability of site for construction are determined mainly by: - Availability of resources - Climatic consideration like (temperature, humidity, rains, wind, storms, etc.) - Physical factors of the site - Topographic consideration - Topographic data - Geological data iii. What is topographic data about? Topographic data is the description of site land form; it includes: - Elevation above sea level - Orientation of the site (e.g., east to southeast, etc.) - Slopes in percentage (0 to 3%, 3 to 8% etc.) - Surface waters (such as ponds, streams, rivers, and drainage patterns) - Hills and valleys (visibility, etc.) - Hood plains, swamps, and quick sand, etc. iv. Which type of buildings should be constructed on rocky sites? Any type of buildings can be construction on rocky sites, but a proper geological study should be conducted first to determine preparation needed Large buildings however are easy to be built on rocks as rocks bearing capacity is very high.

2. Give the classification under which the following buildings fall: (10) a) Museum D1 b) Hospital C2 c) Hotels C1 d) Library (public) D1 e) Departmental stores A1

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature |  | Date | 17/10/2018 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |