Assignment Cover Sheet

| Student Name: | Sean Purcell |
| --- | --- |
| Student Number: | 6196 |
| Course: | Diploma in Construction Management |
| Assignment NO: | 4 Preliminary Investigations, Location and Site Selection |

Marking Criteria:

We expect the learners to write minimum one well expressed point in three lines against each allocated mark. This means one needs to write 15 lines with 5 well expressed points to get high grades for a 5 marks question.

For high grades use examples and illustrations where appropriate.

1. Give short answers to the following questions: (20) i. What is an environmental impact study or EIS? An Environmental Impact Study, or EIS, is a document that describes the impacts on the environment as a result of a proposed Project. It also describes impacts of alternatives as well as plans to mitigate these impacts. When conducting an Environmental Impact Study developers must take into account these factors. Air quality, Water quality and quantity, Solid wastes, Noise, Radiation from nearby radio-active sources, Hazardous conditions, Energy supply and natural resources depletion, Protection of environmentally critical areas i.e. flood plans, wet-lands, beaches and dunes, unstable soil, steep slopes, aquifer recharge areas, Land use in coastal areas, Redevelopment in built-up areas, Density and congestion mitigation, Neighbourhood character and continuity. ii. What is the basic criterion for judging the suitability of the site for construction? The Basic criteria that affect the suitability of a site for all types of building are as follows Availability of physical resources, such as, good water supply, power supply, and material supply, Climatic considerations such as temperature, humidity, solar radiation, rainfall, and wind, Physical factors such as the contour of the site and nature and condition of the soil. iii. What are topographic data about? Topographic data is information about the elevation of the surface of the Earth. Topographic maps are detailed, accurate geographic representations of features that appear on the surface of a specific area. These features include: roads, buildings, urban development, railways, and airports, names of places, geographic features and administrative boundaries. iv. What type of buildings should be constructed on rocky sites? When constructing on Rock Sites the ground for building dams and the rocks below the foundation ground must be impervious to water. Quarries are used to produce crushed stones, building stones, marble for construction work they are also constructed on rocky ground. The main types of buildings Constructed at rocky sites are dams and quarries. 2. Give the classification under which the following buildings fall a) Museum Museums fall under Class D1. b) Hospital Hospitals fall under Class C2. c) Hotel Hotels fall under Class C1. d) Library (public) Libraries fall under Class D1. e) Department Store Department Store falls under Class A1.

Student Statement:

By submitting this assignment, I confirm that this is my own work.

| Student Signature | Sean Purcell | Date | 20-6-2017 |
| --- | --- | --- | --- |

For Tutor / Assessor Use Only

| Total Marks |  |
| --- | --- |
| Marks Obtained |  |
| Percentage / Grade |  |